SSE-PR2 Shure ADC1 Programming

- If not already downloaded, download the Shure Discovery tool from cloud.mc.shure.com/shure-discovery-en

- If not already downloaded, download the Shure Update Utility

- Launch the Shure Update Utility

- Check the IP of the devices in the list and update the computer IP subnet if needed.

- Click Check for Updates or the updates available button

- Scroll down and check the box for any ANI4IN firmware

- Click Download at the bottom

- Once downloaded, close out of that window

- On the main screen firmware tab, click the checkbox next to the ANI4IN

- If the Current Version matches the Version to Install, no update is needed.

- If the Current Version is different than the Version to Install, Press Send 1 Update and wait for the update process to complete.

- Launch the Shure Discovery Tool

- Find the IP of the ANI4IN Audio Network Interface

- Click on or enter the ANI4IN IP into a web browser to access the UI

- The web page will ask to set an admin password.

  - dakpass7

- Enter the new password to login

- Once logged in, on the bottom left click Presets, then from the menu click Import from file

- It will ask to choose an import slot, select Preset 1

- Then press Choose file in the center of the screen

- Open the SSE-xx2 Default Web Config

- Click Import at the bottom of the screen

- The following prompt will pop up on success, click “Go to Load Presets”

- Click on Default Config in slot 1 and Click Load at the bottom of the screen

- Once the preset is loaded, click Settings on the left, and then Network on the top.

- Set Configure IP under Shure Control to Manual and enter the data below.

  - Don’t worry about the Audio Network settings on the right, those will be set by Dante Controller.

- Click Save

- The following prompt will pop up, click Apply Network Settings

Load Dante Preset for SSE-xx2 using Dante Controller.

- Set the computer’s IP to DHCP/Link Local

- Open Dante Controller

- Check that the ANI4IN shows up in the Device Info Tab and is black and shows an IP address.

  - If the device is not showing up, check that the correct network adapter is selected by clicking the Network icon at the top

  - If the device is red, make sure the computer is in the same IP range as the device

- Click the load preset button

- Locate the SSE-xx2 Dante Configuration file and click Open

- Click the SSE-xx2-ADC in Device Roles in this preset

- Click SSE-xx2-ADC -> ANI4IN-xxxxxx (where xxxxxx are random characters) under Devices on the network

- Click Apply Role

- Do the same for the SSR-DM10 in the preset column and DM10 in the devices column  (if trying to program a DM10 as well.)

- Click OK at the bottom

- Click OK on the pop up warning

- Dante Controller will now apply the preset

- When finished it will display Preset Complete

- Reboot the device or rack to finish applying the Dante Settings