# Special instructions for loading the DM-100 Fuelight controller firmware Version 4.5

In addition to following steps 1 through 3 the following information is provided to ensure the Beta Firmware is loaded onto the Programming Board.

This file loaded via the .ttl file will not overwrite the released version of DM-100 if loaded on the programming board.  The beta file will be saved to a different file “slot” on the programming board.

This program, once loaded, will be selectable using the PRGM SELECT switches.

Normally programs will be loaded to the default slot and is selectable with all switches in the DOWN position.

The Beta file will be loaded to the 8th slot and is accessible by moving switch 3 (white text) to the UP position, leaving switch 2 though 0 in the down position.  See below.

Once selected you’ll know you have the right slot if the LCD says: “FL3 DM-100 04.5”

If at any time you wish to go back to version previously loaded, move switch 3 back to the DOWN position.