# RF Explorer Software Installation and Site Survey Instructions

## Step 1: Install RF Explorer Software

Run the above Setup file to install the RF explorer software.

Note on Windows 10 you may be prompted about an unrecognized app.  Click  “More Info” to allow you install the App

Follow the InstallShield Wizard prompts, accepting the term and licensing prompt when asked. Finish the installation, do not Launch the program yet.  We still need to install the USB drivers.

## Step 2: Install the USB drivers

Extract the RFExplorer_USB_Driver.zip file to the location of your choosing.

Inside the newly extracted folder there should be two additional zip files; extract the one matching your current operating system (typically windows 7 or 10)

After extraction run the _x64 or _x86 install file (64 bit or 32 bit…most these days are 64 bit) to install he USB driver.

## Step 3: connect the RFExplorer to the PC/Laptop using the Mini-USB to USB cable (NOT provided)

Note: you do not need to turn the power switch to the ON position.

Wait for windows to finish installing the drivers for the newly detected RF Explorer device.

Ensure the antennas are attached to the analyzer as shown here.

## Step 4: Run the RF Explorer Windows Client App

The program should start up, auto-detect the connected RF explorer and begin to display spectrum data.

Note:  The RF Explorer may be already configured for the proper RF frequencies and additional setup will be required.

## Step 5: Gather the Spectrum Data.

The program will automatically gather many samples of data over time.  Let the Data Sample number increase to ~500 samples and then click the HOLD button to pause samples.  This should give us a pretty good understanding of the local RF spectrum in Fuelight KeyFob frequency range.

- Take a Screenshot (CTRL + Print Screen), or a photo of the screen with a smartphone.  Email the image to Jason.cose@daktronics.com

- In RF Explorer software navigate to File  Save REF Data As…

  - Save a copy of the .rfe file to your local machine and email the file to Jason.cose@daktronics.com