| Revision # | Reason | Revision Date | Revised By |
| --- | --- | --- | --- |
| 0 | Full Release | 10th October 2019 | Pat Begley |
| Enhancements: RM2040-044 This release adds a feature to override the RS232 or RS485 port speeds. RM2040-043 This release addresses bugs in release 042. - NAK when trying to write to Fixed Line. - Fixed !Dc/!cd mix-up. - Added PCB300 Temperature chip read. RM2040-042 This release addresses and partially addresses both design flaws and bugs caused by addressing design flaws in release 041. - Fixed !?? returning rubbish for missing fonts. - Fixed broken LetterTest. - Fixed broken Engineering mode. - Partially fixed Start-up timed-out feature when fixed content not shown timed-out. - Partially fixed !cd command which zapped stored content. Page displayed when !cd received needs to be reset and currently skips on first iteration; as normal on subsequent iterations. RM2040-0041 This release picked up outstanding issues and bugs over a period of six months; so there are quite a few issues addressed. - Hygrosens Probe required - Now allows temperature substitution. (initial version for JC Decaux display) - Page list corrupt. - KillOldQueue 'feature' removed along with subsequent bugs. - GetIO doesn't timeout. - Mended. - CheckAddress removed in Bail(). - Superfluous. - Validate Graphics pixel pairs. - Validation in StashBitmap. '[' does NOT restart packet!!! - modeChangeClearsContent. - Feature added - Western Virginia University reusing mode A. ModeChange function added. - Replace Leading zero fix. - Use control code 0x17 to add a '0' width space - stringLenght -> stringLength. - Because it was annoying me. RM2040-0040 - No Mode 1 unless in config. Set up default Mode 1 if not already done. - Light Sensor message too long. If channel length is 96 or lower use a shorter message string. - Full config not returned. DDX cmd to retrieve full config & sector crc & versions, read crc in LoadConfig and LoadTimeOutCommands RM2040-0039 - CRCs reported incorrectly. MFT data was unnecessarily cleared by SetDefaults. Removed. - Mode 0 allowed in config. DMLx=0 disallowed in LoadConfig. - Superfluous code in DML=. Code removed from LoadConfig. RM2040-0038 - Hardware Staus not working. HardwareStatus - function restored to working version. - GetLEDFailCount not working. GetLEDFailCount - function restored to working version. - Fixed-clock permanent in NVR. SetFixedClock - added !xF0 to remove previous fixed clock without replacement - SilentRemoval bugs fixed - No NextBus DDX support. NextBusDDX support added. Search NextBus for details. RM2040-0032 - Bug fix in letter test routine. Bug: the letter test should use the “internal message mode” (configurable) for both line creation and error checking. The routine was using the correct mode for line creation but not for the error checking (max line that can be created). RM2040-031 - Config buffer pre-clear changed! The buffer is now filled with 0xFF instead of 0x00. The configuration-load process can be stopped by reaching an EOF marker (“.0x00”) or finding 0xFF. Flooding the buffer with 0xFF should ensure the load process ends correctly even if there is an issue with EOF. RM2040-030 - Set Max Scrolls (!ua) added – used to set the number of permitted live scrolls - Query Max Scroll (!?s) added – used to retrieve the number of permitted live scrolls - Max fields per frame/mode increased to 40. - Configurable added for MaxAllowedScrolls. - Bug fix: !zr command caused errors when re-loading the config file. Fix: Pre-clear of the config buffer added to the load config routine. - eeCode added to store MaxAllowedScrolls in NVR. Added commands: Set Max Scrolls Query Max Scrolls Description. Format; !ua. Function; Set the maximum live scrolls allowed on the display at a given time.. Parameter\s; ‘nnn’ number of scrolls permitted (decimal). Related Commands; !?s (Query Max Scrolls). Sample Packet; [0A!ua10]. Default; 10 scrolls. Reply; Standard Reply Format.. NVR; The setting is retained on power cycle. Note. If the sign is double-sided, the setting is related to the master-side of the display. A setting of 10 would mean a total of 20 scrolls are permitted – 10 for the master side and 10 for the slave side. The calculation of live scrolls includes calling-ats and fields that have been forced to scroll because of the scroll-if-too-wide feature.. Bug Fixes: Known Issues: System Qualification: The following components and their respective revision numbers were used in qualifying this firmware release: Component; Revision. |