Setup:

Connect with VNC program or other remote software to SBC (Controller).

Load DMC test tool into SBC in folder on C: drive.

Steps:

- Connect to the SBC's Windows desktop via VNC.

- Navigate to "C:\(Location of DMC test tool folder)

- Double click the DMC Test Tool executable to run it.

- In the window that opens enter 192.168.0.101 port 2001 if it is not already and click Connect.

- Click the Start Test button to retrieve information.

While Start Test is active, information will be actively polled and updated.

- When finished, click Stop Test and then Disconnect and close the DMC Test Tool.

When reading results, the starting xY value is in reference to the address of the DMC card you are connected to (ex. 0Y if there is one card, 1Y if there are 2 cards and you are connected to the second one).  Other values are explained below.

Firmware Version

- Reports the revision of the firmware used by the microcontroller on the display controller. First 4 digits after the FW is the main firmware number, and after the V is the version.  So, in this case main firmware number is 0388 with a revision of 0117 (i.e. major revision 01, minor revision 17).

Temperatures

- Reports the temperature sensed on TS0INTI, TS1EXTI and TS2OPTI. The Read Temperature converts the reading into a readable temperature. This will be in degrees. The temperature is always 7 bytes with a comma separating the three readings.

Temperature Status

- On Firmware 388, this reports if any of the Temperatures are outside of the 3 thresholds set in the EEPROM

Backlight Status

- Reports two items, YY, back light inverter power, and whether the tubes have seen to be lit via the light sensor.

NN = Backlight inverter is powered down, and backlight monitor reports backlight is off.

NY = Backlight inverter is powered down, and backlight monitor reports backlight is on.

YN = Backlight inverter is powered up, and backlight monitor reports backlight is off.

YY = Backlight inverter is powered up, and backlight monitor reports backlight is on.

Light Sensors

- Reports the 2 values from the light sensors. The first value is the percentage of LS0 and the second number is a percentage value for LS1.The numbers returned will be between 0 and 99. This indicates the percentage brightness read from the light sensor(s).

Brightness Level

- Reports the current brightness level of the display. The brightness level may be manual brightness level (that level set over the serial comms) or from the light sensor. The number returned will be between 0 and 99. This indicates the percentage brightness setting used by the display controller to control the LED brightness using a PWM technique. Maximum brightness has been set if returned brightness value is 99. The LED’s will be in the off state if returned value is 0.

PSU Monitor

- Reports the voltages sensed on 5V_MON, 12V_MON and 24V_MON. The Read Voltages converts the reading into a readable voltage. This will be in volts. The voltage is always 5 bytes with a comma separating the three readings. If a PSU is reporting it is bad, then a value of 00.00V will be reported in that cell.